Digital Signal Processing

Final Project

Abdelrahman Mohamed Salama 9224

Pola Nasser 9505

Mohamed Walaa 9574

Part I) ECG Signal Denoising for Telemedicine Applications

## 1.1 ECG Signal Characteristics

An ECG signal records the electrical activity of the heart. It is used by doctors to monitor heart rhythm and detect abnormalities. The recordings used in this project come from the MIT-BIH Arrhythmia Database, sampled at 360 Hz.

A typical ECG waveform has three main parts: the P wave (atrial contraction), the QRS complex (ventricular contraction), and the T wave (ventricular recovery). The QRS complex is the most important , it has sharp, high peaks used to detect heartbeats, so preserving its shape is a priority.

The useful ECG frequency range is 0.5 Hz to 100 Hz. Anything below 0.5 Hz is noise from slow drift, and anything above 100 Hz is high-frequency muscle noise.

## 1.2 Identification of Noise Types

Three types of noise affect ECG signals:

Baseline wander (below 0.5 Hz) a slow drift caused by breathing or body movement. It shifts the whole signal up and down. Since it is well below the useful ECG band, a high-pass filter removes it cleanly.

Power-line interference (50 Hz) a sinusoidal hum from the electrical power supply. It sits inside the useful ECG band, so it cannot be removed with a simple HPF or LPF. A narrow notch filter centered at 50 Hz is the correct solution.

Muscle noise / EMG (above 100 Hz) random high-frequency noise from muscle activity near the electrodes. Since the useful ECG band ends at 100 Hz, a low-pass filter handles this.

| Noise Type | Frequency Range | Filter Required |
| --- | --- | --- |
| Baseline wander | Below 0.5 Hz | High-pass filter |
| Power-line interference | 50 Hz (narrow) | Notch filter |
| Muscle noise (EMG) | Above 100 Hz | Low-pass filter |

1.3 Filtering Objectives

- Remove baseline wander using a high-pass filter with passband starting at 0.5 Hz

- Suppress power-line interference using a narrow notch at 50 Hz

- Reduce muscle noise using a low-pass filter with cutoff at 100 Hz

- Preserve the QRS complex shape and timing

- Minimize phase distortion to avoid shifting ECG peaks

- Compare FIR and IIR designs in terms of order, phase, and computational cost

1.4 Filter Specifications

| Specification | Value | Justification |
| --- | --- | --- |
| Sampling frequency | 360 Hz | Fixed by the MIT-BIH Arrhythmia Database |
| Useful ECG bandwidth | 0.5 – 100 Hz | The clinically significant ECG components (P wave, QRS complex, T wave) are all located within this band. |
| HPF passband edge | 0.5 Hz | The lowest useful ECG component begins at 0.5 Hz; the filter must pass everything above this frequency. |
| Notch center frequency | 50 Hz | Matches the power-line frequency used in Europe and many other regions where the database was recorded. |
| Notch quality factor (Q) | 50 | A high Q = 50 produces a very narrow notch (bandwidth = 50/50 =1Hz), minimizing distortion of surrounding ECG components. |
| LPF passband edge | 100 Hz | Passes all diagnostically meaningful ECG content; anything above 100 Hz is outside the useful ECG band. |
| LPF stopband edge | 150 Hz | A 50 Hz transition band is wide enough to achieve the required attenuation at a practical Chebyshev filter order. |
| Max FIR Filter Order | 500 | FIR Filters requires high order |
| Max IIR Filter Order | 4-6 | IIR Filters requires low order |
| Passband ripple | ≤ 1 dB | Limits amplitude distortion within the useful ECG band, preventing waveform deformation that could impair diagnosis. |
| Stopband attenuation | ≥ 40 dB | Reduces noise power to less than 1% of its original level, which is sufficient for clinical ECG quality. |
| Transition bandwidth | Practical / moderate | A very narrow transition band would require an excessively high filter order, increasing computational cost and latency. |
| Phase Distortion | minimized | Zero Phase filtering using filtfilt can be used to avoid ECG peak shifting. |
| Latency | As low as possible | Useful for telemedicine |

1.5 Implementation Constraints

Computational cost : higher order means more calculations per sample. This matters for battery-powered wearable devices.

Latency: FIR filters delay the signal by N/2 samples. At 360 Hz, a high-order FIR can introduce hundreds of milliseconds of delay, which is a problem in real-time systems. IIR filters have much lower latency.

Phase distortion: FIR filters have linear phase, meaning all frequencies are delayed equally and the ECG shape is preserved. IIR filters have non-linear phase, which can distort peak positions. For offline analysis, this can be fixed using filtfilt, but that is not suitable for real-time use.

1.6 Filter Limitations

1.6.1 FIR High-Pass Filter

Very high order required: The cutoff of 0.5 Hz is only 0.00278 of the Nyquist frequency. A FIR filter needs order N = 500 to achieve adequate roll-off at such a low normalized frequency, resulting in 501 multiply-accumulate operations per sample.

Large latency: A causal FIR of order 500 introduces a group delay of N/2 = 250 samples = 694 ms at 360 Hz. This is unacceptable for real-time telemedicine. The filtfilt workaround used here eliminates the delay but requires the full signal to be buffered and cannot be used in real-time systems.

Startup transient: The filter output is unreliable during the first N = 500 samples (approximately 1.4 seconds), which corrupts the first few heartbeat cycles.

1.6.2 FIR Notch Filter

Cannot achieve a truly narrow notch: A FIR band-stop filter places zeros on the unit circle but has no poles to restore gain immediately outside the notch. Achieving Q = 50 (bandwidth = 1 Hz) with a FIR requires extremely high order. The IIR iirnotch achieves the same with a 2nd-order filter by placing a pole pair just inside the unit circle at the same angle as the zeros, cancelling the zero effect for all other frequencies.

Wide transition bands distort ECG: The FIR band-stop gradually attenuates frequencies from 47–49 Hz and 51–53 Hz, potentially distorting high-frequency QRS components. The IIR notch attenuates only a 1 Hz band and recovers to 0 dB gain almost immediately outside it.

Computational overhead: Using 501 taps to do what a 2nd-order IIR performs with 5 coefficients is a 100:1 inefficiency. This is the primary reason iirnotch is the standard choice for power-line rejection in all practical biomedical devices.

1.6.3 IIR Filter Limitations (Butterworth and Chebyshev)

Non-linear phase: IIR filters delay different frequency components by different amounts, distorting relative timing between the P wave, QRS, and T wave. Chebyshev is worse than Butterworth in this regard due to its equiripple design. filtfilt is used here to compensate, but this prevents real-time use.

Chebyshev passband ripple: The 0.25 dB equiripple causes non-uniform gain across 0.5–100 Hz, slightly altering relative amplitudes of P wave versus QRS peak, which matters in clinical amplitude measurements.

Part II) Multi-Band Digital Speech Equalizer

## 1. Objective

The objective of this project is to design and implement a multi-band digital speech equalizer using MATLAB for enhancing speech clarity in podcasts and recorded audio. The system allows frequency-selective amplification and attenuation of speech components to improve intelligibility and reduce noise influence.

## 2. Methodology

The system divides the audio signal into multiple frequency bands and processes each band independently using either FIR or IIR filters. Each band is assigned a gain in decibels (dB), which is applied after filtering. The processed bands are then recombined in the time domain to reconstruct the enhanced speech signal.

The workflow includes:

- Reading the input audio signal.

- Splitting the signal into predefined frequency bands.

- Designing bandpass filters (FIR or IIR).

- Filtering each band separately.

- Applying gain to each band.

- Summing all bands to reconstruct the output.

- Normalizing and analyzing the final signal.

## 3. Frequency Band Design Justification

The selected frequency bands are designed according to speech characteristics:

- 0–100 Hz: Contains rumble and environmental noise, attenuated

- 100–300 Hz: Low vocal energy, contributes to muddiness, slightly reduced

- 300–800 Hz: Fundamental speech components, slightly boosted

- 800–2000 Hz: Primary speech intelligibility range, significantly boosted

- 2000–5000 Hz: Consonant clarity region, strongly boosted

- 5000–10000 Hz: Speech brightness and articulation, moderately boosted

- 10000–20000 Hz: Airiness and high-frequency detail, slightly boosted

This configuration improves intelligibility while reducing low-frequency noise interference.

## 4. Filter Design Choice (FIR)

A FIR filter with Hamming window was selected for this implementation due to:

- Guaranteed stability (non-recursive structure)

- Linear phase response, preserving waveform shape

- Reduced phase distortion in speech signals

- Suitable for audio processing applications where signal integrity is important

However, FIR filters require higher computational cost compared to IIR filters, especially at higher orders.

## 5. Gain Strategy (dB Scaling)

Gain values were chosen to enhance speech clarity while suppressing noise:

- Low-frequency bands were attenuated to reduce rumble.

- Mid-frequency bands (300 Hz – 5 kHz) were amplified to enhance speech intelligibility.

- High-frequency bands were slightly boosted to improve consonant clarity.

The use of logarithmic gain (dB scale) allows perceptually meaningful control over audio intensity.

## 6. Results and Observations

Time Domain

The processed signal maintains the original structure of speech while exhibiting increased amplitude in speech-active regions.

Frequency Domain

Energy redistribution is observed from low-frequency regions toward mid and high-frequency speech bands.

Power Spectral Density (PSD)

The PSD analysis shows a reduction in low-frequency energy and an increase in power within the 800 Hz – 5 kHz range, confirming successful equalization.

Spectrogram

The spectrogram reveals improved visibility of high-frequency components corresponding to consonants and speech articulation.

## 7. FIR vs IIR Trade-offs

Although only FIR was implemented in this run, the theoretical comparison is as follows:

FIR Advantages:

- Linear phase response (no phase distortion)

- Always stable

- More suitable for speech preservation and clarity

FIR Disadvantages:

- Requires higher filter order

- Increased computational complexity

IIR Advantages:

- Lower computational cost

- Efficient implementation for real-time systems

IIR Disadvantages:

- Non-linear phase response (possible speech distortion)

- Stability must be carefully controlled

## 8. Artifacts and Distortion Discussion

Some minor artifacts may appear due to:

- Strong gain applied in mid/high frequencies

- Filter transition band overlap between adjacent bands

- Accumulated noise amplification in high-frequency region

However, normalization reduces clipping effects and maintains overall signal stability.

## 9. Listening Evaluation

Subjective evaluation shows:

- Improved speech intelligibility

- Clearer consonant sounds (“s”, “t”, “k”)

- Reduced low-frequency rumble

- Slight increase in perceived brightness of audio

Overall, the equalized signal is easier to understand compared to the original recording.

## 10. Conclusion

The implemented multi-band equalizer successfully enhances speech clarity by selectively controlling frequency bands relevant to human speech perception. The system demonstrates effective spectral shaping, resulting in improved intelligibility and reduced noise influence. The approach is suitable for podcast enhancement, lecture recordings, and general speech processing applications.