Digital Signal Processing

Final Project

Abdelrahman Mohamed Salama 9224

Pola Nasser 9505

Mohamed Walaa 9574

Part I) ECG Signal Denoising for Telemedicine Applications

## 1.1 ECG Signal Characteristics

An ECG signal records the electrical activity of the heart. It is used by doctors to monitor heart rhythm and detect abnormalities. The recordings used in this project come from the MIT-BIH Arrhythmia Database, sampled at 360 Hz.

A typical ECG waveform has three main parts: the P wave (atrial contraction), the QRS complex (ventricular contraction), and the T wave (ventricular recovery). The QRS complex is the most important , it has sharp, high peaks used to detect heartbeats, so preserving its shape is a priority.

The useful ECG frequency range is 0.5 Hz to 100 Hz. Anything below 0.5 Hz is noise from slow drift, and anything above 100 Hz is high-frequency muscle noise.

## 1.2 Identification of Noise Types

Three types of noise affect ECG signals:

Baseline wander (below 0.5 Hz) a slow drift caused by breathing or body movement. It shifts the whole signal up and down. Since it is well below the useful ECG band, a high-pass filter removes it cleanly.

Power-line interference (50 Hz) a sinusoidal hum from the electrical power supply. It sits inside the useful ECG band, so it cannot be removed with a simple HPF or LPF. A narrow notch filter centered at 50 Hz is the correct solution.

Muscle noise / EMG (above 100 Hz) random high-frequency noise from muscle activity near the electrodes. Since the useful ECG band ends at 100 Hz, a low-pass filter handles this.

| Noise Type | Frequency Range | Filter Required |
| --- | --- | --- |
| Baseline wander | Below 0.5 Hz | High-pass filter |
| Power-line interference | 50 Hz (narrow) | Notch filter |
| Muscle noise (EMG) | Above 100 Hz | Low-pass filter |

1.3 Filtering Objectives

- Remove baseline wander using a high-pass filter with passband starting at 0.5 Hz

- Suppress power-line interference using a narrow notch at 50 Hz

- Reduce muscle noise using a low-pass filter with cutoff at 100 Hz

- Preserve the QRS complex shape and timing

- Minimize phase distortion to avoid shifting ECG peaks

- Compare FIR and IIR designs in terms of order, phase, and computational cost

1.4 Filter Specifications

| Specification | Value | Justification |
| --- | --- | --- |
| Sampling frequency | 360 Hz | Fixed by the MIT-BIH Arrhythmia Database |
| Useful ECG bandwidth | 0.5 – 100 Hz | The clinically significant ECG components (P wave, QRS complex, T wave) are all located within this band. |
| HPF passband edge | 0.5 Hz | The lowest useful ECG component begins at 0.5 Hz; the filter must pass everything above this frequency. |
| Notch center frequency | 50 Hz | Matches the power-line frequency used in Europe and many other regions where the database was recorded. |
| Notch quality factor (Q) | 50 | A high Q = 50 produces a very narrow notch (bandwidth = 50/50 =1Hz), minimizing distortion of surrounding ECG components. |
| LPF passband edge | 100 Hz | Passes all diagnostically meaningful ECG content; anything above 100 Hz is outside the useful ECG band. |
| LPF stopband edge | 150 Hz | A 50 Hz transition band is wide enough to achieve the required attenuation at a practical Chebyshev filter order. |
| Max FIR Filter Order | 500 | FIR Filters requires high order |
| Max IIR Filter Order | 4-6 | IIR Filters requires low order |
| Passband ripple | ≤ 0.25 dB | Limits amplitude distortion within the useful ECG band, preventing waveform deformation that could impair diagnosis. |
| Stopband attenuation | ≥ 40 dB | Reduces noise power to less than 1% of its original level, which is sufficient for clinical ECG quality. |
| Transition bandwidth | Practical / moderate | A very narrow transition band would require an excessively high filter order, increasing computational cost and latency. |
| Phase Distortion | minimized | Zero Phase filtering using filtfilt can be used to avoid ECG peak shifting. |
| Latency | As low as possible | Useful for telemedicine |

1.5 Implementation Constraints

Computational cost : higher order means more calculations per sample. This matters for battery-powered wearable devices.

Latency: FIR filters delay the signal by N/2 samples. At 360 Hz, a high-order FIR can introduce hundreds of milliseconds of delay, which is a problem in real-time systems. IIR filters have much lower latency.

Phase distortion: FIR filters have linear phase, meaning all frequencies are delayed equally and the ECG shape is preserved. IIR filters have non-linear phase, which can distort peak positions. For offline analysis, this can be fixed using filtfilt, but that is not suitable for real-time use.